# Laboratory 3A:

# Fourier Transformation and FFT

In this laboratory you experiment with some properties of the Fourier Transformation such as duality and time-bandwidth product. Plus, you will gather further experience with applications of the numerical algorithm Fast Fourier Transformation (FFT).

Exercise 1: Working with the USB-Scope

We use an USB-Scope and Signal generator for the following experiments. Let us start with a simple measurement to understand how this device works.

| Step | Setup | Hint |
| --- | --- | --- |
| 1 | Start a new workspace and select a scope and a wave-generator - Workspace > New - Drag & Drop selected windows |  |
| 2 | Set the wave-generator for a single sinus with fsig = 1KHz |  |
| 3 | Connect the output of the wave-generator to one input channel of the scope |  |
| 4 | Observe the signal in the time and frequency domain | - Scope: Default pane - Scope: FFT pane |
| 5 | Observe the following snapshots and answer to the question | Unfortunately for HS20 we still do not have USB-scopes for the students to take home. Therefore copied the measurement snapshots below. Plus you can remotely control the scope. |

Snapshot-A: Full Range

- Consider the indication in the top of the time domain window:

What are the implications for the measurement in the frequency domain using the FFT?

- What is the resolution of the spectrum measurement? How do you need to change the measurement setup in order to confirm your answer?

- Which harmonics do you expect in this spectrum? How do you need to change the measurement setup in order to confirm your answer?

- Are there also unexpected harmonics? Which ones? How do you need to change the measurement setup in order to confirm your answer?

Snapshot-B: Zoom in frequency domain

- What do you need to change in the settings to the time domain measurement to achieve a finer frequency resolution in the spectrum plot? Explain your answer with a short calculation.

Exercise 2: Duality Property of the Fourier Transformation

- Add a 2nd channel in the wave generator and describe a custom waveform with a sinc shape.

- Once you save your custom wave-shape, you can now select the desired parameters, like: frequency, amplitude, …

Once enabled the wave-generator will output a periodic sinc shaped pulse, with the selected parameters.

Snapshot-C: Periodic Sinc-Pulse

- Before you turn to the next page, think about what do you expect to observe in the FFT window of the scope, when observing this periodic sinc pulses?

Hint: Check now the description of the duality property of the Fourier Transformation (in chapter 3 of the script),

Snapshot-D: Periodic Sinc-Pulse in the Time and Frequency Domain

- Which parameter do you need to change in the wave-generator to decrease the DC-value and get closer to a square shaped spectrum? Make the necessary changes and verify the result.

- Control now the spectrum plots below, and relate to the signal characteristics in the time domain like: frequency and width of the lobes or zero-crossings.

Snapshot-E: Periodic Sinc-Pulse Zoom-out

Snapshot-F: Periodic Sinc-Pulse Zoom-mid

Snapshot-G: Periodic Sinc-Pulse Zoom-in

Exercise 3 : Time-Bandwidth Product

Read now the description of the FT property Time-Scaling or Time-Bandwidth product. How can you confirm this property by changing one of the settings from the current measurement? Try out your supposition.

Hint : in order to easily observe the changes in the bandwidth, fix the frequency range which you want to observe. For example start=0Hz and stop=50kHz.

# Exercise 4 Tchin-Tchin: Synthesis of a Glass-Sound.

In this exercise you analyze and synthesize a copy of a given glass sound.

Take the recorded sound example in file Glas.mat, and conduct the following steps:

- Analyze the audio signal in the recorded sound-file using the FFT;

- Generate a plot of the audio signal in time and in frequency domain;

- Complete the Matlab script, which synthesizes the glass-sound by adding up the few dominant frequency components, and weighting the signal with an envelope in the time domain.

Tip-1 : Use the exp() function to implement the envelop curve;

Tip-2 : The file sisy1_lab3A_exer_synthesis.m is available as a start point.