Laboratory 3A:

# Fourier Transformation and FFT

Sample Solution

Exercise 1: Working with the USB-Scope

- Resolution in frequency domain:  fstep = 800k / 8192 ~ 100Hz   (precisely  97.65… Hz)

- Calculated resolution above. Can change time-base to check. Please do so.

- Expect a single harmonic at 1KHz (or as close as resolution allows it).

- With large frequency range (0-400KHz) seeing 2 peaks around 250kHz. Believe these are image spectra, around a sampling frequency of 250kHz. We will speak of image spectra in chapter 4.

If you change time-base for a larger observation window and smaller freq. range, you address the following question.

Here we observe some power at 3kHz, which corresponds to the 3rd harmonic, about 60dB below the 1st harmonic level. This happens often when for example, an amplifier presents some non-linearity (e.g. smaller gain closer to rail values) and generates some extra harmonics.

Exercise 2: Duality Property of the Fourier Transformation

- OK.

- Pay attention that the x range (from/to) will determine the slice of the time function, which will be periodically repeated by the generator.

- Thinking about the duality property, expect a somehow square shape in the frequency domain (amplitude spectrum).

- Change the offset in the waveform generator (see snapshot below), to reduce DC-value in the spectrum, and better visualize the square shaped spectrum.

Time Domain			Frequency Domain

==============================================================

Snapshot-E :		periodic		=> 	discrete

T0 = 1ms		=>	f0 = 1kHz

==============================================================

Snapshot-F :		periodic “sinc-shape”  => 	discrete “square-shape”

T0 = 1ms			f0 = 1kHz

The “” means that the shapes are not perfect.

For example, the sinc is limited (clipped), and

the square gets correspondingly less sharp edges.

==============================================================

Snapshot-G :		lobe-width=50us	=> 	bandwidth = 1/50us=20kHz = [-10k, +10kHz]

In the spectrum measurement we see the positive frequencies, therefore 0-10kHz.

Obs.: with the current selected window for FFT, we see a bit wider BW. Change the window of FFT to see its influence.

Exercise 3 : Time-Bandwidth Product

Several possibilities. Liked a lot this one below done by students:

# Exercise 3 Tchin-Tchin: Synthesis of a Glass-Sound.

Observation:

- Major tones and relative amplitudes are the main characteristics perceived by human hearing. Relative phase difference of major tones not relevant, and this explains why point to point difference between synthesized and original sound can be so big, although both sounds perceived as similar.

- Interesting: adding some low-pass noise, makes synthesized sound quite difficult to distinguish from original one.

% SiSy1 Prak3A - Aufgabe Klang Synthese

% =================================

clear all, close all, clc;

% __1__Load and hear to reference sound

load Glas

sound(y,Fs)

% __2__Check loaded variables and declare aux / time / freq vectors

%    Fs     [Hz]

%     y      [vector with amplitude values over time]

Ts = 1/Fs;              % seconds

N = length(y);         % number of points in registered sample

aux = 0:1:N-1;        % index vector

t = Ts*aux;             % time vector

f = (Fs/N)*aux;       % freq vector

% __3__Plot registered sound in time

figure(1)

subplot(221), plot(t,y), grid on, hold on;

xlabel('time [s]')

% Model envelope as decaying exponential

max(y)*0.37

tau = 0.235;     % check where curve decayed to 0.37*initial value (approx exp(-t/tau) )

x_env = max(y)*exp(-t/tau);

figure(1)

subplot(221), plot(t,x_env,'r--','LineWidth',2), hold off

legend({'recorded','envelope'})

% __4__Analyse registered sound in frequency

Y = (1/N)*fft(y);                       % fast-fourier-transform

Y_dB = 20*log10(abs(Y));      % amplitude in dB

% __5__Plot registered sound in frequency

figure(1)

subplot(222), plot(f,Y_dB), grid on, hold on;

pause(2)

axis([0 Fs/2 max(Y_dB)-60 max(Y_dB)+2])

xlabel('frequency [Hz]')

legend('Dominant Tones')

%%

% Synthesize sound with:  (sum of sinus) * (envelope curve)

% Look for first 6 dominant peaks, within max-40dB

[pksdB,locs] = findpeaks(Y_dB(1:N/2),'NPeaks',6,'MinPeakHeight',max(Y_dB)-30,'MinPeakDistance',10,'MinPeakProminence',2);

figure(1)

subplot(222), plot(f(locs),pksdB,'go'), hold off;

f(locs)                                            % check frequency value for peaks selected

AmpdBr = pksdB-pksdB(1)    ;      % take amplitude values relative to 1st peak

AmpLir=10.^(AmpdBr/20);            % calculate amplitude in linear scale

% Sum up sinus-signals (each with respective frequency & amplitude)

x=zeros(1,N);

for id=1:1:length(locs)

x = x + AmpLir(id)*sin(2*pi*f(locs(id))*t);

end

x = x/max(x) ;  % normalize for max(amplitude)=1

% Shape the sum of sinus with envelope curve

x_synth = 1.15*(x_env.*x);

%    tuned amp (with 1.15) to better match volume-impression of recording

figure(1)

subplot(223), plot(t,x_synth,'k', t,x_env,'r--'), grid on

xlabel('time [s]')

legend({'synthesized','envelope'})

% Play Synthesised Sound and Replay original for comparison

sound(x_synth,Fs);

pause(2)

figure(1)

subplot(224), plot(t, (y-x_synth),'g'), grid on

xlabel('time [s]')

legend('Difference')

pause(2), figure(2)

plot(t,y,'b',t,x_synth,'k',t,(y-x_synth),'g--'),grid on

xlabel('time [s]'), legend({'recorded','synthesized','difference'})

pause(2), axis([0 0.01 -0.4 0.4])

%% Add band-limited noise (background)

x_spr = x_synth + 0.02*lowpass(rand(1,N)-0.5,0.4);

sound(x_spr,Fs);

pause(4)

sound(y,Fs);

pause(2), figure(2)

plot(t,y,'b',t,x_synth,'k',t,(y-x_synth),'g--',t,x_spr,'m-.'),grid on

xlabel('time [s]'), legend({'recorded','synthesized','difference','synth+noise'})

pause(2), axis([0 0.01 -0.4 0.4])